DEPARTMENT OF ENVIRONMENT  AND NATURAL RESOURCES	Sheet __1__ of __2__

DIVISION OF ENVIRONMENTAL HEALTH	PROPERTY ID #: Jameson Way; PID#102325

ON-SITE WASTEWATER SECTION	COUNTY: Lincoln

SOIL/SITE EVALUATION

for ON-SITE WASTEWATER SYSTEM

OWNER: Mars 5 Holdings LLC		 	                                                                                       APPLICATION DATE:____________

ADDRESS: PO Box 1535 Denver, NC 28037	                                                DATE EVALUATED: 4/2022

PROPOSED FACILITY: Commercial Site w/ 150 employees	         PROPOSED DESIGN FLOW (.1949): 2250 GPD	      PROPERTY SIZE: 11.14 Acres

LOCATION OF SITE: Jameson Way Denver, NC	PROPERTY RECORDED: 2/19/2020

WATER SUPPLY:    Private        Public          Well        Spring       Other ______________________________________________

EVALUATION METHOD:   Auger Boring      Pit     Cut                TYPE OF WASTEWATER:      Sewage    Industrial Process    Mixed

| P R O F I L E # | .1940 LANDSCAPE POSITION/ SLOPE % | HORIZON DEPTH (IN.) | SOIL MORPHOLOGY (.1941) |  | OTHER PROFILE FACTORS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | .1941 STRUCTURE/ TEXTURE | .1941 CONSISTENCE/ MINERALOGY | .1942 SOIL WETNESS/ COLOR | .1943 SOIL DEPTH | .1956 SAPRO CLASS | .1944 RESTR HORIZ | PROFILE CLASS & LTAR |
| AB 1 | L 13-14 % | 0"-25" | R C SBK | SS SP FR SEXP |  | 48” | Few to common saprolite inclusions 25”-48” |  | 0.3 |
|  |  | 25"-48" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
| AB 2 | L 13-14 % | 0"-6" | R SCL WSBK | SS SP FR SEXP |  | 48” | Few to common saprolite inclusions 45”-48” |  | 0.3 |
|  |  | 6"-45" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 45"-48" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
| AB 3 | L 13-14 % | 0"-4" | Br CL WSBK | SS SP FR SEXP |  | 48" | Few to common saprolite inclusions 29”-48” |  | 0.3 |
|  |  | 4"-29" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 29"-48" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
| AB 4 | L 13-14 % | 0"-13" | YBr SCL WSBK | SS SP FR SEXP | Lithochromic colors included. | 48" |  |  | 0.3 |
|  |  | 13"-48" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
| AB 5 | L 13-14 % | 0"-6" | YBr SCL WSBK | SS SP FR SEXP | Ybrs at 35”+ | 48" |  |  | 0.3-0.25 |
|  |  | 6"-48" | R C SBK | SS SP FR SEXP |  |  |  |  |  |

| DESCRIPTION | INITIAL SYSTEM | REPAIR SYSTEM | OTHER FACTORS (.1946): _____________________________________ SITE CLASSIFICATION (.1948): ________________________________ EVALUATED BY: Michael Athanas and Associates____________________ OTHER(S) PRESENT: _________________________________________ |
| --- | --- | --- | --- |
| Available Space (.1945) | 6750 FT2 | 22500 ft2 |  |
| System Type(s) | 25% Reduction | Pretreated Drip |  |
| Site LTAR | 0.25 | 0.1 |  |

LEGEND

use the following standard abbreviations

SOIL	        CONVENTIONAL         LPP	     MINERALOGY/

LANDSCAPE POSITION            GROUP                         TEXTURE   	            .1955 LTAR*        .1957 LTAR*            CONSISTENCE                    STRUCTURE

CC (Concave Slope)                             I		S (Sand)			1.2 - 0.8	            0.6 - 0.4	NEXP (Non-expansive)            G (Single Grain)

CV (Convex Slope)				LS (Loamy Sand)					SEXP (Slightly Expansive)      M (Massive)

D (Drainage Way)										EXP (Expansive)	           CR (Crumb)

DS (Debris Slump)                               II		SL (Sandy Loam)		0.8 - 0.6                  0.4 - 0.3			           GR (Granular)

FP (Flood Plain)				L (Loam)								           SBK (Subangular Blocky)

FS (Foot Slope)												           ABK (Angular Blocky)

H (Head Slope)                                    III		Si (Silt)			0.6 - 0.3	           0.3 - 0.15			           PL (Platy)

L (Linear Slope)				SiCL (Silty Clay Loam)						                               PR (Prismatic)

N (Nose Slope)				CL (Clay Loam)

R (Ridge)					SCL (Sandy Clay Loam)			                          MOIST                                       WET

S (Shoulder Slope)				SiL (Silt Loam)

T (Terrace)										                     VFR (Very Friable)		NS (Non-sticky)

IV		SC (Sandy Clay)		0.4 - 0.1                  0.2 - 0.05	FR (Friable)		                     SS (Slightly Sticky)

SiC (Silty Clay)					FI (Firm)			S (Sticky)

C (Clay)						VFI (Very Firm v. Very Sticky)              VS (Very Sticky)

O (Organic)		  None		None	EFI (Extremely Firm)		NP (Non-plastic)

SP (Slightly Plastic)

*Adjust LTAR due to depth, consistence, structure, soil wetness, landscape, position, wastewater flow and quality.		                     P (Plastic)

NOTES                                                                                                                                                                                                                                                          VP (Very Plastic)

HORIZON DEPTH		In inches below natural soil surface

DEPTH OF FILL		In inches from land surface

RESTRICTIVE HORIZON	Thickness and depth from land surface

SAPROLITE		S(suitable) or U(unsuitable)

SOIL WETNESS		Inches from land surface to free water or inches from land surface to soil colors with chroma 2 or less - record Munsell color chip designation

CLASSIFICATION		S (Suitable), PS (Provisionally Suitable), or U (Unsuitable)

Evaluation of saprolite shall be by pits.

Long-term Acceptance Rate (LTAR): gal/day/ft2

Show profile locations and other site features (dimensions, reference or benchmark, and North).

DENR (######)

Review (#####)

DEPARTMENT OF ENVIRONMENT  AND NATURAL RESOURCES	Sheet __2__ of __2__

DIVISION OF ENVIRONMENTAL HEALTH	PROPERTY ID #: Jameson Way; PID#102325

ON-SITE WASTEWATER SECTION	COUNTY: Lincoln

SOIL/SITE EVALUATION

for ON-SITE WASTEWATER SYSTEM cont.

| P R O F I L E # | .1940 LANDSCAPE POSITION/ SLOPE % | HORIZON DEPTH (IN.) | SOIL MORPHOLOGY (.1941) |  | OTHER PROFILE FACTORS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | .1941 STRUCTURE/ TEXTURE | .1941 CONSISTENCE/ MINERALOGY | .1942 SOIL WETNESS/ COLOR | .1943 SOIL DEPTH | .1956 SAPRO CLASS | .1944 RESTR HORIZ | PROFILE CLASS & LTAR |
| AB 6 | L 13-14 % | 0"-6" | YBr SCL WSBK | SS SP FR SEXP |  | 43” | Few to common saprolite inclusions 33”-43” PS |  | 0.3 0.1+ Drip Repair |
|  |  | 6"-33" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 33"-43" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 43"-48" |  | SS SP FR SEXP |  |  |  |  |  |
| AB 7 | L 13-14 % | 0"-6" | YBr SCL WSBK | SS SP FR SEXP |  | 48” | Few to common saprolite inclusions 33”-48” |  | 0.3 0.1+ Drip Repair |
|  |  | 6"-33" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 33"-48" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
| AB 8 | L 13-14 % | 0"-30" | R C WSBK | SS SP FR SEXP |  | 33" | Few to common saprolite inclusions 30”-33” PS |  | 0.3 0.1+ Drip Repair |
|  |  | 30"-33" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 33"-48" | V L Saprolite M | SS SP FR SEXP |  |  |  |  |  |
| AB 9 | L 13-14 % | 0"-3" | YBr SCL WSBK | SS SP FR SEXP |  | 48" | Few to common saprolite inclusions 29”-48” |  | 0.3 |
|  |  | 3"-29" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 29"-48" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |
| AB 10 | L 13-14 % | 0"-4" | YBr SCL WSBK | SS SP FR SEXP |  | 48" | Few to common saprolite inclusions 32”-48” |  | 0.3 0.1+ Drip Repair |
|  |  | 4"-32" | R C SBK | SS SP FR SEXP |  |  |  |  |  |
|  |  | 32"-48" | R CL WSBK | SS SP FR SEXP |  |  |  |  |  |