TRIAD

# BUILDING STATISTICS

Location:			The Triad Building

105 Despatch Drive, East Rochester NY  14445

Year Built:			1993

Building Type:	Pre-Engineered metal and decorative block on 6” concrete slab; insulated and tinted glass windows from grade to 9 ft. above grade; 2 ft. x 2 ft. exterior block columns; 25 ft. x 80 ft. bays; loading docks, 4 ea. truck high with overhead door

Zoning:	Industrial

Occupancy Rating:	Business Group B; Factory Industrial Group F

Land:	3.0 Acres

Stories:	One

Roof line:     18’ to 20”-8”

Underside of joist:     15’ to 18’

Ceilings – suspended grid:     9” office, other areas vary

Floor Area:	Gross Rentable:  31,250 square feet; no interior common area

Partitioning:	Existing Tenant Layout – Metal Studs and Drywall

Parking:	130 spaces – lighted

Roof:	Standing seam, metal deck

HVAC:	Heating and cooling using split systems, each system having a gas-fired furnace along with cooling coils and an air cooled compressor.  Individually metered

Fire Protection:	Fire Sprinklers throughout; electrically monitored fire sprinkler alarm

Security:	Each rentable space has its own entrance and exits.  Entrance and exit doors are provided with integral looks, pneumatic closure, and keyed with a 5 pin key.

Flooring:  	Carpeting, Ceramic Tile and Vinyl Composition Tile

Lighting:	2’ x 4’ 18 cell Parabolic Fluorescent troffers, and Recessed Lighting

Electric Service:	1 ea. 800 amps; 120/208 – 3 phase

Telephone Service/Fiber	Incoming Lines – 500 pair / First Light, 300 mbps/300 mbps

Partitions:	Steel Studs / drywall

Finishes:	Paint; Wallpaper; Wood Trim; Glass