| Location Sampled/Material | Result | Lead Hazard Levels |
| --- | --- | --- |
| Bare Soil Near Playground and Picnic Tables | 230 ppm | ≥ 400 ppm |
| Bare Soil from Yard/Non-Play Area | 1400 ppm | ≥ 1200 ppm |
| Bare Soil from Dripline of Residential Building facing 45th Street | 5300 ppm | ≥1200 ppm |
| Bare Soil from Walkway Leading from Main Gate to Residential Building Facing Ursuline Street | 770 ppm | ≥400 ppm |
| Bare Soil from Dripline of Residential Building facing Ursuline Street | 7900 ppm | ≥1200 ppm |
| Nestle Nido Milk Powder | Non-Detectable | NA |
| Dust Wipe Blank | Non-Detectable | NA |
| Floor Dust Wipe at Back Door in Residential Building Facing Ursuline Street (1ftx1ft) | 160 ug/ft2 | 10 ug/ft2 |
| Floor Dust Wipe at Front Door in Residential Building Facing Ursuline Street (1ftx1ft) | 120 ug/ft2 | 10 ug/ft2 |
| Water Samples from Kitchen Taps in Residential Buildings and Water Fountain in Office | Non-Detectable | 15 ppb |

The results of the XRF analysis determined that all original components of the buildings contained lead-based paint (LBP). *See the Excel file for specific locations, components, and results of XRF sampling.